Crucorney Woodland group health and safety policy to deliver safe systems of work, health and safety responsibilities and safe working methods.

JC 15/10/2024 V4

This document sets out the following:

- How the group will provide and maintain a safe environment and enable safe systems of work for carrying out its objectives

- Defines health and safety responsibilities and safe working methods.

These two aspects of the CEG’s health and safety policies are identified in the Groups Health and Safety Policy Document, which was reviewed in 25/5/2018 and signed by the then group chairman David Jackman.

There are a number of activities undertaken by the group which have significant health and safety risks.  Those activities, the associated risks and how they can be managed are set out below:

- Tree felling and cross cutting including winch use

- Firewood processing including splitting, stacking and transport of firewood

- Charcoal kiln loading, firing and emptying

There are a number of aspects to effective health and safety risk management that are common to all aspects of the group’s work.  They include:

- A risk manager to be identified for each work session who will be responsible for ensuring activities are carried out in accordance with the policy, including ensuring a first aid kit is available for each work session.

- Risk manager to assess operator competency in use of hand tools and to provide guidance if required.

- Each operator is responsible for their own safety and for the safety of others

- The wearing of appropriate personal protective equipment

- Ensuring tools are in good condition

- Reporting near misses and accidents for inclusion in the accident book which are reviewed as part of the risk briefing before every work group starts

- Listening carefully and attentively to the risk manager during the safety briefing and putting questions to the risk manager for any aspect requiring clarification.

- Applying a ‘so far as is reasonably practical’ approach to risk management

- No-one under the age of 18 to attend a work group.

- Only those who have read and agreed to abide by the contents of this risk management document to take part in the activities listed above.

- First aid kits to be checked annually and to be taken to each work location.

- A qualified fist aider to be present at all work groups, and all volunteers are encouraged to take a fist aid course when available

- All operatives to carry charged mobile phones during work sessions.

- No single working, all operatives to work as a minimum in pairs, except where operational needs makes single working imperative e.g late night change of chimneys on kiln

Tree felling and cross cutting:

Main risks:

- Serious injury or death from falling trees or limbs of trees during felling operations.  A risk exacerbated by ash die back disease.

- Serious injury or death from chainsaw use and winch operation.

Risk management:

- All tree felling and cross cutting to be carried out by qualified operators in accordance with specified training regimes and current codes of practice including, for example, warning signs on access routes, wearing of hi vis clothing and personal protective equipment

- separate felling groups being at least two tree lengths apart

- where a winch is employed, a lead operator ensuring chain-sawing and winching are undertaken in a controlled and coordinated manner. The lead operator to be trained in both chainsaw and winch operation

- block and tackle employed to ensure the winch operator is not at risk from the tree being felled.

- When cross cutting ensuring that any tension or compression in felled timber is understood prior to cutting

- No-one to be within two metres of a chainsaw operative unless there is a significant risk to the operator.

Firewood processing including splitting, stacking and transport of firewood

Main risks:

- Serious injury or death from chainsaw use to cut billets into firewood lengths

- Serious injury or death from a splitter head coming loose from a shaft or shaft breakage or splitter coming out of operator’s hands.

- Serious injury or death from splitting wedge flying out from a billet when struck.

- Serious injury to lower limbs to someone using a splitter from a misdirected or glanced splitter swing

- Crush injuries from being trapped by either tractor or trailer, either from overturning or between the tractor/trailer and trees

- Limb or head injuries to tractor driver or those travelling in the trailer from tree branches above or adjacent to the track

- Damage to feet and lower limbs from tractor or trailer tyres.

- Trips and falls due to brambles, particularly when carrying a billet

- Injury to eyes, head, body or lower limbs from protruding branches, either on the ground or still attached to the tress, especially if branch has sharp broken ends.

- injury resulting from stack collapse

- Back and limb injuries from attempting to move a trunk which is too heavy for one operator to move alone

- Damage to fingers from being trapped when stacking or loading billets, cut firewood or when inserting a wedge

- Serious eye injuries from wedge material spalling from the wedge head during use.

Risk management:

- Only qualified chainsaw operators to be within 2m of a chainsaw operator cutting billets to length.

- All splitters to be checked for soundness prior to use by a competent person, including personal splitters

- Splitting to only be undertaken by those deemed experienced enough by a competent person.  Those without sufficient experience to be trained by an experienced splitter and then monitored to ensure compliance with trainer advice

- Those using a splitter to ensure that, should the head become detached from the shaft, or the shaft break, or splitter comes out of operator’s hand, no-one is likely to be struck.  A distance of at least 10m to be achieved between splitter operation and other operatives in the direction of swing.

- Gloves NOT to be worn by splitter users.

- Stout footwear to worn by all those involved in firewood processing, stacking and transport of firewood, preferably with steel toe caps.

- Tractor driver to be responsible for the safety of all those in the vicinity of the tractor and trailer from use of the tractor and trailer during tractor maneuvering and use of the three point linkage. Engine turned off and hand brake engaged when tractor driver isn’t seated on the tractor.

- Main routes between cross cut timber and stacks to be cleared of brambles

- Stacks to be no more than 1.5m high and billets to be at least 1.2m in length stacked squarely.

- All operators to advise colleagues if they see an operator attempting to lift a trunk or billet that is excessively heavy

- All operators to take care when loading billets onto the stack or trailer so as to avoid injuring another operator

- All wedges to be checked for spalling on a regular basis and any excess material removed.

Charcoal kiln loading, firing and emptying

Main risks:

- Serious injury or death from chainsaw use to cut timber to length for use in the kiln

- Damage to back and limbs from removal or replacement of the kiln lid,  and transporting cut timber to and lifting into the kiln

- Trapping fingers when removing or replacing the kiln lid

- Damage to fingers and limbs from careless placing of cut timber into the kiln.

- Tractor and trailer use is part of kiln operations.  See above for risks and their mitigation associated with tractor and trailer use.

- Use of splitting maul as part of kiln operations. See above for risks and mitigations.

- Burns from using lit diesel fuel to ignite the kiln and from hot kiln during charcoal burning

- Fire risk to surrounding vegetation ignited from kiln operation particularly during periods of dry weather.

- Lung and skin damage from inhaling charcoal dust during charcoal extraction

- Appropriate breathing apparatus for charcoal extraction in the kiln and dust masks to be employed during bagging operations.

Risk management:

- Only qualified chainsaw operators to undertake chain sawing and no others to be within 2m of chainsaw operations

- Risk manager to appoint a competent person to be allocated to role of kiln leader for loading and unloading

- Adequate numbers of operatives to remove kiln lid and to load and unload kiln

- Fire extinguisher to be available during kiln lighting and operation

The undersigned confirms they have read this document and understands both risks and how to manage them.

JC 27092024.